10 CONSTRUCTION SITE PUBLIC PROTECTION

10.1 PURPOSE

A brick mason’s perimeter scaffold collapsed resulting in a severe injury to a pedestrian. A section of a temporary construction fence tipped over during a heavy wind and injured a pedestrian. Can the concerned contractors be held liable for these losses? The answer is a “yes.”

Any construction site carries risks associated with its activities that can cause injury/illness to the members of the public and damage to their properties. Construction contractors can be held liable for injury, death, or property damage to the members of the public arising from actions of their employees or subcontractors. This liability is commonly known as public liability or third-party claims.

The cost of dealing with such lawsuits and claims can be significant. In addition, public liability claims can create an adverse publicity to the contracting firm. It is important that contractors understand, in addition to their workers, they also have a duty to protect the members of the public and their property during a construction project. Hence, public protection should be an integral part of the contractor’s site-specific safety plan (SSSP). This section will provide some guidance to contractors to identify construction site exposures and industry’s best practice controls to minimize or eliminate injuries and property damage to the public.

For the purpose of this section, “public” will include project visitors, tour groups, homes and businesses within or adjacent to construction sites, guest, delivery drivers, commercial visitors, and the general public. In other words, all persons not employed by or not under contract with the contractors working on a construction site.

10.1 RESPONSIBILITY AND CONTRACTS

With a few exceptions, the general contractor (GC) will have the ultimate responsibility for control of the project site, and to make sure all subcontractors take appropriate measures to protect the public. This responsibility should be included in the contract between the GC and the subcontractors and their tiers.

  - SAFETY/ EMERGENCY PLANNING

Prior to starting construction operations, the GC should consider and address all potential risks associated with the project activities that may lead to public injury or property damage. Public protection should be an integral part of every construction activity. Hence, all project-related job hazard analysis (JHA) and pre-task plans (PTP) should consider public protection. In addition, the project should develop an emergency action plan, which is in compliance with DOSH and other applicable regulatory requirements. See Chapter 17 for detailed information about emergency management. The plan should address at a minimum the following emergencies:

- Flood, hurricane, tornado, serious inclement weather

- Fire

- Electrical outage

- Chemical leak or spill

- Medical emergency, fatality, multiple injury accident

- Catastrophe or collapse

- Crime against property on the jobsite

- Crime against persons on the jobsite

Instruct personnel on the jobsite about the emergency procedures. A specific plan to address public contempt or protest that deals with members of the public who purposely place themselves or others at risk by failing to observe or heed warnings or other directives or safety precautions. For example, the plan may require notification to agencies with authority to control public activities, e.g., the police or fire department, and cessation of all construction activities that may cause a hazard until the public is controlled. A specific plan for handling threats or other violence as follows:

- Bomb or arson threats

- Threats of violence to the construction site employees

- Suicide attempts/threats

- Others

The plan should Include immediate notification to and interface with appropriate authorities.

10.3 INSURANCE

GCs should ensure that they have adequate insurance coverage to protect them against claims made by members of the public who suffers injury or property damage in connection with the project. GCs should also require subcontractors to include them as additional insured under their general liability policies. Contractors should also consider risks associated with vendors and suppliers, and ways to manage those risks.

10.4 TRAINING

Public protection should be one of the main topics covered during the new employee site-specific orientation. Workers and supervisors should be trained in the various site-specific public risks, and the mitigation measures. The training should also include incident reporting procedures and emergency response procedures.

  - PUBLIC COMMUNICATION

GCs should take a proactive approach when it comes to communicating construction impacts with the neighboring homes and businesses. Property owners do not like surprises. Hence, proper communication about the project scope and schedule will build good relationships with the neighbors. Create a project telephone hotline or email to receive project-related inquiries. Some contractors, depending on the project size and public risks, have created a position or designated a team member as the project’s minimum impact coordinator (MIC) to manage third-party risks. On many projects, this role is assigned to the project safety manager. The project should share MIC’s contact information with the neighboring property owners, and let them know to contact the project in case of an issue or concern. Some of the important tasks of the MIC should include:

- Act as a public liaison, by serving as the main point of contact and respond to public inquiries and complaints on matters of public interest and concern.

- Coordinate, attend, and present to business, community, and neighborhood group meetings regarding the project, its impacts, and intended mitigations. Focus on how the project will deal with noise issues, traffic disruptions, dust issues, etc.

- Monitor project’s hotline voicemail and maintain a log of incoming inquiries and response.

- Create and distribute project-related public education materials, and be available to attend community events and meetings.

- Prepare and distribute advertisement materials to the neighbors.

- Investigate any accidents related to the public and implement corrective actions.

  - PRIVACY

Sometimes the construction will take place in proximity to neighboring properties. On one project, a hospital close to the project had patient rooms facing the building site where construction workers can easily see through the patient rooms. These privacy issues should be identified and mitigated prior to starting construction.

  - PUBLIC PROTECTION RISKS AND CONTROLS

Some of the most common public risks associated with construction projects, and best practices to control them are discussed in the following sections.

10.7.1 Site Security

What safety and site security measures should the project have in place? The magnitude of site security measures varies from project to project. Several factors determine the level of security needed for the project such as location (rural vs. urban), type of structure (building vs. roadways), size, duration, level of attractive nuisance exposures (e.g., close to school), level of crime in the area and a few others. Some things to consider:

- Designate a person responsible for security to monitor site security including periodic inspection of fencing and signage. The person should also review site security measures on a regular basis since conditions change on the site frequently. Contractors have hired off-duty police officers or retired police officers to serve as site security managers.

- Where appropriate, the project sites should have adequate perimeter control with the help of a temporary fencing. The main objective of the fence is to avoid unauthorized or inadvertent access to the project site. The fence should be strong enough and high enough, and properly secured and anchored to prevent tipping. There have been many incidents where pedestrians were struck by the falling fence, which resulted in lawsuits against contractors claiming negligent erection and maintenance of construction fencing.

- The perimeter fence gates should have locks, which should be locked at the end of shift and proper access controls during and after hours of operations. Site gates should be closed when not in use and opened only when required for special deliveries or other authorized entries.

- Post appropriate warnings and signs conspicuously on the fence, gates, and any other site entrances such as “no trespassing and keep out – construction area,” to warn the public.

- Establish a system in place to control vehicle access in and out of the site. Only authorized vehicles should be allowed on site. This is critical when working near hospitals when patients or elderly people might get easily confused and enter job sites by mistake.

- Set up an approval system in place to remove or alter fencing. During the course of the project, contractors remove or alter the fence to carry out their scope of work. This practice should be strongly discouraged or should only be allowed after consultation with the project supervision.

Issue gate keys only to authorized supervisors and establish a system in place to control key issuance.

- When site access control with a fence is not possible or not appropriate, discourage site access using physical barricades, warning ropes, and signage.

- Take pictures at the end of the shift of the safe and secure site conditions is the best practice to protect against lawsuits

- Inform local law enforcement agencies of your work schedule and security measures, and request them to patrol the site after working hours.

- Other site security measures that have been successfully used by contractors in conjunction with fencing include but not limited to:

  - Security guard service during non-work hours both weekdays and weekends

  - Live security camera surveillance with motion sensors

  - Site under surveillance signage

  - Intrusion alarm systems or tracking devices tied to a monitoring service

Figure 10-1: Example of properly installed construction site security fence (Picture Courtesy of XXX)

Figure 10-2: Example of fence warning signage (Picture Courtesy of XXX)

10.7.2 Attractive Nuisance

All construction sites are an attractive nuisance, which can attract people to the project to explore it. With its fleet of heavy equipment, tower cranes, mobile cranes, trucks, hiding places, dirt piles; construction sites, in particular, are very attractive to children and teenagers. If they are hurt while, on the site, contractors can be held liable for their injury. Consequently, the project should take all due-diligence (standard of care) and good faith efforts to safeguard the project. Hence, if site access control fails (fence or other barriers), in order to avoid liability, the following precautions should be taken to prevent injuries:

- Secure storage areas containing hazardous materials and waste

- Protect open holes and leading edges including manholes and excavations by cordoning the area off with ropes or even physical barriers

- Ensure the stability of material storage areas

- Employ tools and equipment controls

- Restrict access to tower crane towers, electrical rooms, etc.

- Remove keys from the heavy equipment such as excavators, pile drivers, lifts, etc.

- Protect entry into confined spaces

- Protect and prevent access to unfinished stairways, scaffolds, etc., with physical barriers

- Remove unattended ladders that can provide access to unprotected areas

10.7.3 Visitor Controls

How can the project avoid or reduce potential liability associated with visitors? The project should create a visitor access policy. The project should:

- Install signs at the project entrance and gates that instruct all visitors to report to the project office.

- Maintain a visitor log; all visitors should sign-in and out at the project office.

- Require all visitors to sign a “hold-harmless” agreement holding the contractors/owners harmless in case of injury or damage.

- Develop and deliver a modified site orientation and document the orientation. The orientation should, at a minimum, cover the high hazard areas, designated walkways, site Attire and Personal Protective Equipment (PPE) requirements.

- All visitors must wear the required PPE while on the project site. If the visitor fails to comply with the requirements, deny site access.

- Never allow visitors to tour the site by themselves, always provide an escort who is familiar with the site.

- Develop a visitor badge or visitor pass system for ease of identification of the visitor. Some contractors have even used a special hard hat for visitors (e.g., red hard hats).

- Sometimes school children or college students tour sites for educational purposes. Tour groups can be complicated compared to individual visitors; extreme precaution must be taken.  Establish some age limit for tours. For example, children under the age of 12 will not be permitted to accompany tours or visit the site. Also in case of children, ensure an adult sign the hold-harmless agreement, preferably a parent or teacher.

Figure 10-3: Example of visitor signage at the project entrance that instructs all visitors to report to the project office (Picture Courtesy of XXX)

10.7.4 Traffic Control

Failure to identify and control exposures related to traffic control can lead to significant losses in the form of injuries and public liability to both heavy civil contractors (i.e. highway work zone) and building contractors (i.e. city streets adjacent to jobsites). Some common exposures include pedestrians walking by or near the project and motorists driving by the project. Some of the best practices to follow to avoid losses related to traffic are:

- Implement approved traffic control plans for compliance with applicable local standards. It is best practice to create both an onsite and offsite traffic control plans.

- Use of certified flaggers to control traffic when working in highway work zones, and for directing vehicles in and out of jobsites

- Proper segregation between traffic and pedestrian routes

- Backup alarms use on trucks and construction equipment

- Good use of clean and clear signs and barricades in compliance with applicable local standards

- Daily inspection of traffic control devices - many times traffic control devices that are blown onto vehicles have led to public injuries. When responding to lawsuits involving highway work zone accident, attorneys will check to see if proper control was in place.

- Daily inspection to ensure the roads are free of “ground hazards” created by construction work such as potholes, gravel, steel trench plates, and other debris. Install concrete wash-out and tire wash areas to prevent from mud and rocks migrating to the streets. There have been numerous incidents involving bicyclists slipping on rocks from construction sites on the streets or a broken windshield due to these rocks.

Figure 10-4: Example of Traffic Control – Highway Site (Picture Courtesy of XXX)

Figure 10-5: Example of Traffic Control - Urban Site (Picture Courtesy of XXX)

10.7.5 Property Protection

What are the various properties that should be considered? How to protect against property damage? Property adjacent to construction sites can be damaged, and its occupants can be put at risk. The property can include:

- Adjacent buildings or structures (neighboring homes and business, city-owned sidewalks, city streets, etc.)

- Adjacent lands

- Vehicles of others (cars driving by the site, cars parked near the site)

- Tools and equipment of others

- Business interruption related losses

The project team should consider all potential sources that can cause property damage. The construction activities that can cause the most exposure include:

- Crane and rigging work can cause loads to shift and damage adjacent properties

- Compressed gasses, flammable and combustible liquids stored on site can cause an explosion.

- Open flame hot work can cause site fire or smoke damage

- Demolition activities can cause dust, vibration, flying objects, and noise issues

- Vibrations can result from activities such as demolition, blasting, pile driving, drilling, tunneling can cause adjacent property collapse or damage

- Painting activities, concrete pours, etc., can cause material escape from the site and cause damages to passing traffic, pedestrians, and adjacent properties

- Ground movement caused by excavation, compaction, dewatering, undermining, tunneling, shoring, etc., can cause adjacent property settlement or damage

- Some example incidents,

  - An excavation contractor’s backfill and subsequent compaction caused adjacent property settlement

  - A painting contractor’s overspray caused a parked black car turn into an orange car

  - An overspray from a concrete hose from a high-rise damaged the windshield of a car driving by on the city street below

  - A crane load struck a vehicle driven by a member of public close to the job site

Even though, construction activities can sometimes lead adjacent property owners suddenly to notice cracks in walls and foundations, many of these, however, could be pre-existing. A preliminary pre-construction property third-party survey is highly recommended to document existing conditions of adjacent structures and surrounding areas prior to commencement of construction. The conditions should be continuously monitored as construction progresses. Photographs, videos, physical measurements, etc., should be taken to establish pre-existing conditions clearly. It is highly recommended to conduct a similar post-construction survey to compare with the pre-existing conditions. If the construction activity did cause some damage, it might be beneficial to fix it. Ensure there are physical and fire separation and protection measures for adjacent properties.

A pre-task plan of all activities should consider any potential public property damage and provide control measures is in place. For example, an excavation contractor digging a trench in an area known to have a city water line. The water line supplies water to the entire neighborhood. What precautions should be taken? Any underground or overhead utility service strike can lead to injury or death, and will affect the business and residents. The contractor should identify the existing water line using the “know what’s below – CALL before you dig.” In addition, the contractor can also use “ vacuum excavation” to get a visual on the line before starting to dig with an excavator to avoid utility strike that will affect the residents. For crane and rigging tasks, develop and implement good hoisting and material handling programs

Any open flame hot work or smoking that could cause a fire damage to adjacent properties. The contractor should develop and implement a good fire prevention and protection program.

10.7.6 Pedestrians

Areas around a construction site pose several hazards to the pedestrians. Some of the common pedestrian hazards include but not limited to uneven or slippery surfaces, steep surfaces, unprotected holes and edges, overhead obstructions, poor pedestrian routes, inadequate lighting and many others.

It is best practice to close sidewalks adjacent to projects when necessary with proper directional signage to channel the pedestrians safely around the construction site per applicable local standards. Certain states and cities have specific requirements when closing sidewalks.

Maintain the pedestrian walkways free of slip, trip, and fall hazards. Ensure the walkways are kept clean at all times by removing any debris and warn pedestrians about any change in elevation of the walking surfaces. Provide adequate lighting for the walkways. When appropriate use overhead protection with the help of covered walkways. Temporary sidewalks, ramps or stairs should be provided with guardrails on both sides.

Where it is necessary to provide alternative or restricted pedestrian routes, make sure you address the needs of persons who are blind or partially sighted, use wheelchairs, or have other mobility difficulties. Temporary pedestrian walkways should have uniform surfaces, with access ramps, handrails if necessary, good lighting and a suitable surface like smooth non-slip/trip concrete.  Clear signs should be provided to direct people to the correct pedestrian route, particularly if the destination building has been obscured by the construction site.  Clearly define the walkway by continuous barrier and ensure the walkway is kept free of debris and clutter.

Figure 10-6: Example of Pedestrian pathway around construction sites (Picture Courtesy of XXX)

Figure 10-7: Example of Pedestrian pathway around construction sites (Picture Courtesy of XXX)

10.7.7 Other hazards

This paragraph lists other common hazards that could cause personal injuries.

Falling Objects

Many construction activities creates a potential hazard of falling objects such as material handling work along the building perimeter, concrete pours, construction debris that can get wind-borne such as loose plywood or cut metal decks, that can cause serious injury or death to members of the public. Is the area subject to strong winds? These hazards can be controlled by protection systems such as:

- Safe and proper material handling procedures so as, not to interfere with any adjacent structures

- Adequate overhead protection

- Adequate edge protection

- Physical separation of the site from the public

- Crane safety procedures during heavy winds

Slips, trips, and falls

- Properly maintained surfaces

- Well protected holes and edges

- Adequate provision of safe pedestrian routes

- Adequate site lighting on pedestrian routes

- Slip-free surfaces

Radiation

- Control of radiation risks is impacting public. For example, how to protect public from welding UV light?

- Installation of proper signage

Noise

- Compliance with local noise ordinance

- Coordinate start time and schedule of high noise activities with adjacent property owners to minimize impacts. For example, concrete pour at 3 a.m., can generate a lot of noise concrete truck back-up alarms, light towers might shine at the neighbors window, etc

Fire

- Good fire prevention and protection program

- Smoking controls

Environmental Hazards

- Maintain a clean work area with adequate debris removal

- Develop and implement an airborne contaminant management plan to prevent the escape of a hazardous substance to escape from the site such as silica, asbestos, etc

- Approved erosion and sediment control plan

- Approved hazardous materials and waste management plan

- Daily inspection of the site to contain the environmental hazards

Figure 10-8: Example of Falling Object Protection (Picture Courtesy of XXX)

Additional Resources, Readings, and References

- ANSI/ASSE A10.34-2001 (R2012) – Protection of the Public on or Adjacent to Construction Sites - One of the key element of the standard is a public hazard control plan (PHCP). The PHCP is a guideline to aid the contractor in identifying, evaluating, preventing or minimizing their exposures as identified in the standard.