21 SAFETY INSPECTIONS

21.1 PURPOSE

Safety inspections are a significant hazard identification tool on a construction site, which will help prevent accidents by correcting those hazards. This section is intended to provide step-by-step best practice guidance to contractors to develop a new or improve an existing site-specific inspection program.

The following are the objectives of the safety inspection program:

- Reduce the risk of injury or loss to project personnel, property or the quality of work products through the identification and elimination of safety and health deficiencies

- Provide an ongoing, systematic monitoring mechanism to measure compliance with relevant regulatory or contract standards

- Involve the project team members in safety and health inspection and surveillance

- Identify areas of potential risk and loss

- Identify opportunities for safety and health education and training

- Check and audit past training and skill development

- Identify and develop positive attitudes for safety and health

21.2 DEFINE PROGRAM PURPOSE

Define and communicate the purpose of the site’s safety inspection program to all worker through site orientation. The workers should realize and be confident that inspections are not to “get them,” but is a proactive accident prevention measure. The primary goal of safety inspections should be to identify and correct (a) unsafe conditions, (b) unsafe acts and (c) deficiencies in the Site Specific Safety Plan (SSSP). Include safety inspections as one of the major elements of the Site Specific Safety Plan (SSSP).

21.3 IDENTIFY INSPECTION NEEDS

The SSSP should identify the project-specific inspection needs by reviewing:

- Project contract requirements (e.g., Does the owner require special inspection?)

- Applicable regulatory requirements (e.g., DOSH competent person scaffold inspection)

- Company’s accident history from similar projects (e.g., WMSD risk factors inspection)

- Manufacturer recommendations for all equipment and tools on the site (e.g. Annual crane inspections)

21.4 DEVELOP INSPECTION RECORDING TOOLS

Develop safety inspection tools such as checklists and forms to record the inspection findings (Exhibit 21-1). Checklists are typically created by making a list of site-specific hazards (unsafe acts/conditions). It is an excellent tool for inspection of specific items such as:

- Construction equipment inspections (e.g., scissor lift, concrete pump truck, crane)

- Emergency equipment and supplies (e.g., fire extinguisher, first aid kit)

- Personal Protective Equipment (e.g., Full-body harness)

- Trade- or task-specific inspection (e.g., excavation inspection or landscape contractor)

- Health hazards measurement and testing (e.g., measurement of noise exposure)

- Inspection checklist or form focusing on fall, struck by, caught in or between and electrocution hazards

Beware, since checklists consists of pre-determined hazards or items to inspect, there is a potential that inspectors could miss “unrecognized” hazards not listed on the list. In addition, since these checklists are generic, ensure you modify these checklists to fit your specific jobsites. Use open-form inspections for general site-wide inspections (Exhibit 21-2). Contact entities such as your company’s workers’ compensation insurance carrier, industry associations (e.g., AGC), labor unions, DOSH; who can provide free pre-made safety inspection checklists for construction.

21.5 PLAN FOR INSPECTION

Identify individuals who will conduct safety inspections, the frequency of inspections, and level of formality of these inspections. Ensure that those identified to conduct inspection are adequately trained to recognize hazards. Safety inspections can be classified into formal and informal inspections.

- Informal – Inspections that are conducted by all site personnel including craftsmen as part of their job responsibilities throughout the day. Typically, informal inspections do not involve documentation such as forms or checklists. However, they do play a crucial part of an SSSP by helping to identify safety issues on a daily basis.

- Formal – Inspections that are conducted by certain individuals or entities, which are scheduled at regular intervals. It typically involves the use of inspection tools for recording inspection findings. The frequency of formal inspections can vary from weekly to annually, depending on the potential for injury or damage, (or) regulatory requirements associated with the jobsite, a specific task, or equipment. For example, electrical hot work might require frequent inspections of the work area.

Based on industry best practices, depending on project-specific factors, the following inspections and timeline is recommended for “general” site inspections (see Table 21-1).

Table 21-1: Safety Inspection Frequency and Level of Formality

| Who should conduct inspections? | Frequency? | Formality? |
| --- | --- | --- |
| Workers & field supervisors (e.g., superintendents, foreman) | Daily | Informal |
| Project management (e.g., project managers (PM) | Monthly | Formal |
| Owner representative (e.g., owner PM, safety) | Monthly | Formal |
| Contractor(s) top management | Quarterly | Formal |
| Site Safety Committee | Bi-weekly | Formal |
| Site safety professionals | Weekly | Formal |
| Site safety professionals | Daily | Informal |
| DOSH consultation walks | As-needed | Formal |
| Insurance loss control experts (random) | Quarterly | Formal |
| DOSH compliance audits | * | Formal |
| Regulatory agency mandated inspections | ** | Formal |
| Manufacturer or underwriter mandated inspections | ** | Formal |
| Tools and Equipment | Before use | In/formal |

*Depends on imminent danger, fatality/catastrophe, complaints/referrals, and programmed

**Refer all applicable standards and regulations

***Refer operators manual

Note: The site inspection requirements should be communicated to all subcontractors through pre-bid and pre-construction meetings and the contracts.

21.6 ASSEMBLE INSPECTION KIT

Safety inspectors should be set up for success with an inspection kit. The kit may include, depending on the project, clipboards, inspection checklists, forms, pens, flashlights, camera, tape measure, electrical testing equipment, and monitoring/sampling devices (i.e. sound level meter, light meter). However, paper-based inspections might become history in a few years. Many companies are using software called “apps” installed on portable electronic devices such as smartphone or tablet computers to conduct safety audits and inspections. These safety inspection apps even come with a feature to take pictures during the inspection and add them to the report, and email a copy of the full report to selected subcontractors. Some apps can measure sound and light levels, and also provides a quick reference of standards and regulations on your fingertips – making inspections more efficient. Refer Chapter 8 of this handbook for more information on smartphone apps application in safety management.

21.7 TRAIN THE INSPECTORS

New supervisors or safety committee members, may have never conducted an inspection. Hence, it is important to train the inspectors on the following topics:

- Proper inspection techniques – proper preparation, conducting the inspection and follow-up for corrective actions.

- Addressing workers performing unsafe acts

- Common site hazards and controls

- Familiarity with applicable standards and regulations

- Familiarity with company/site policies including the requirements of the SSSP

21.8 CONDUCT THE INSPECTIONS AND RECOMMEND CORRECTIVE ACTIONS

The inspector should take adequate time to conduct a thorough review of the entire jobsite. Depending on the jobsite, it might take an hour to 4 hours to complete an effective inspection. Most hazards can be fixed during the inspection by working with the foreman in charge of the work area or workers associated with the issue. Try to correct the cause of the unsafe condition or act, to prevent reoccurrence. Some items might require disciplining workers, which will trigger the site’s progressive discipline program. Hence, safety issues that were not or cannot be fixed during the inspection (e.g., worker operating scissor lift without training), should be noted on the inspection form along with other details (i.e. work location, concerned subcontractor).

At the end of the inspection make a separate list of safety issues with your proposed solutions and attach a deadline for each corrective action. Distribute the “action list” to the concerned subcontractor’s key supervisors as soon as possible, and require them to fix it before the deadline. Note activities or tasks that are posing imminent danger conditions should be stopped, and the safety issue should be fixed immediately.

Figure 21-1 Safety Professional Performing Scaffold Inspections using I-pad (Courtesy Hensel Phelps Construction)

21.9 FOLLOW-UP FOR CORRECTIVE ACTION

Did the proposed solutions were implemented? Inspections are useless if the safety issues identified are not fixed. The inspector should periodically check to see the progress towards the corrective action until the safety issues are resolved.

21.10 FILE THE INSPECTION FORMS

Once all safety issues are resolved, make a note on the form that should include (a) what corrective action was taken and (b) when and who completed the action. Keep a copy in the jobsite file. It is called closing the loop. It is a very important step, especially when responding to a lawsuit related to that jobsite. It will demonstrate the company’s due diligence in maintaining a safe and healthy work environment, and compliance with its safety program.

21.11 IDENTIFY TRENDS

Inspections are one of the most common safety leading indicator that can predict accidents by showing “trends” in unsafe acts and conditions. For example:

- 80% of the workers observed were not wearing appropriate gloves. Is there a problem with the project’s glove policy?

- 60% of the scissor lifts operating on site did not have a spotter. Is there an enforcement issue with spotter requirements?

Reviewing the inspection reports for trends and addressing them in a timely manner will prevent accidents.

21.12 SHARE LESSONS LEARNED WITH ALL SITE PERSONNEL

Hazard trends identified during inspections offer an excellent training opportunity. Communicate the inspection results and trends to the workers at all-hands or mass safety meetings. Review inspection results at the safety committee meeting as well. Encourage site supervisors to share them during their pre-task plan meetings. It will allow workers to watch for these hazards and unsafe work practices, and report them immediately to their immediate supervisor.

21.13 RECORD AND SHARE BEST PRACTICES

The number of contractors on a jobsite will vary from 1 to 100, each implementing various safety inspection best practices. On certain projects workers might suggest innovative inspection practices, if they were successful, make sure it is carried over to another project. Consider the following list of programmatic best practices developed based on the experience of major construction contractors.

    - Worker Safe Behavior Observation (WSBO) Program

The WSBO is a leading indicator metric that generates a weekly WSBO project score (0-100), with the help of the following steps:

- Observe worker behaviors who are part of the crew or work area for a short period. Make a minimum of 30 observations per week to get a meaningful sample.

- Calculate a WSBO score, which is the ratio of workers with safe behavior to the total number of employees observed (e.g., 80% of crew members were safe). Set project goal, for example, the average WSBO score goal for the project is 95.

- Record and tabulate the “frequency” of the nature of unsafe behavior (e.g., no safety glass) from all observations for that week.

- Analyze trends and implement corrective actions.

- Continue the process to check the effectiveness of the corrective actions and identify new trends, if any.

    - Monthly Management Inspection (M2I) Program

The M2I is another leading indicator metric that generates a weekly M2I project score for each subcontractor or the project as a whole, with the help of the following steps:

- Create a standard safety inspection form.

- Require a monthly audit by the project manager to ensure compliance with project safety requirements and applicable regulations. In addition to a job walk, the audit should include a review of accident reports, safety inspections, site recordkeeping, etc.

- Start the audit with a score of 100. Deduct points for each non-compliance items. Add points (extra credit) for items that are above and beyond requirements.

- Compute the net score.

- Use the M2I score to reward subcontractors with superior performance and require subcontractors with inferior score to provide corrective actions.

- Track and make efforts to improve M2I score.

    - Safety Management by Walking Around (SMBWA)

The SMBWA is an management inspection technique that involves the project managers of the owner, general contractor and subcontractors walking around on the jobsite, directly observing the site operations. Management should engage their workers in conversations, and receive feedback about the effectiveness of the site safety program, and try to resolve any deficiencies in a timely manner. During these walks, the managers should “recognize safe behaviors” and coach to “modify unsafe behavior.” The SMBWA technique shows management commitment to worker safety and builds employee morale.

Additional Resources, Readings, and References

- Oregon OSHA Sample Inspection Checklists: http://www.orosha.org/standards/checklists.html

- Texas department of Insurance Sample Construction Safety Inspection Checklist: Construction Safety Inspection Checklist: http://www.tdi.texas.gov/pubs/videoresource/cklgenconstru.pdf

Exhibit 21-1: Example Project Safety Inspection Closed Form

(Used by Permission from GLY Construction)

| ##### Superintendent: |  |  | ##### Date: |  |
| --- | --- | --- | --- | --- |
| ##### Foreman: |  |  | ##### Job Name: |  |
| ##### # GLY Employees: |  |  | ##### Job #: |  |
| ##### # Sub Employees: |  |  | ##### Project Mgr: |  |

| # COMPLIANCE |  |  |  | CONTRACTOR | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| Yes | No | # JOB OFFICE REQUIREMENTS |  |  |  |
|  |  | # Substance Abuse Testing |  |  |  |
|  |  | # Posting Requirements |  |  |  |
|  |  | # Signage |  |  |  |
|  |  | # MSDS Manual |  |  |  |
|  |  | # Safety Training (Hire-in Booklet) |  |  |  |
|  |  | # Tool Box Meetings |  |  |  |
|  |  | # Daily Logs |  |  |  |
|  |  | # Job Hazard Analysis |  |  |  |
|  |  | # Flex & Stretch |  |  |  |
|  |  | # LEED Recycle Log |  |  |  |
| Yes | No | # TOOLS |  |  |  |
|  |  | # Power Tools, Wiring & Grounding |  |  |  |
|  |  | # Protective Equipment |  |  |  |
|  |  | # Safety Devices |  |  |  |
| Yes | No | # PERSONAL PROTECTIVE EQUIPMENT |  |  |  |
|  |  | # Personal Clothing |  |  |  |
|  |  | # Goggles or Face Shield |  |  |  |
|  |  | # Hard Hats |  |  |  |
|  |  | # Protective Clothing |  |  |  |
|  |  | # Body Harness & Lanyard |  |  |  |
| Yes | No | # COMPANY POLICY |  |  |  |
|  |  | # Hard Hats |  |  |  |
|  |  | # Eye Protection |  |  |  |
| Yes | No | # FIRE PROTECTION |  |  |  |
|  |  | # Extinguishers on Site |  |  |  |
|  |  | # Fire Watch or Extinguishers |  |  |  |
|  |  | # Tested & Inspected Extinguishers |  |  |  |
| Yes | No | # MATERIAL HANDLING EQUIPMENT |  |  |  |
|  |  | # Ladders |  |  |  |
|  |  | # Scaffolding |  |  |  |
|  |  | # Man Lifts |  |  |  |
|  |  | # Fork Lifts |  |  |  |
|  |  | # Equipment Checklist |  |  |  |
| Yes | No | # HOUSEKEEPING |  |  |  |
|  |  | # Aisles & Stairs |  |  |  |
|  |  | # Floors |  |  |  |
|  |  | # Storage & Piling of Material |  |  |  |
|  |  | # Lighting & Ventilation |  |  |  |
| Yes | No | # SAFE PRACTICES |  |  |  |
|  |  | # Hazardous Material Containment |  |  |  |
|  |  | # Working under Suspended Loads |  |  |  |
|  |  | # Removing Machine Tool Guards |  |  |  |
|  |  | # Handrails & Guardrails |  |  |  |
|  |  | # Floor Openings Covered & Marked |  |  |  |
|  |  | # Access & Ramps |  |  |  |
| Yes | No | # FIRST AID/CPR |  |  |  |
|  |  | # Certified Provider on site |  |  |  |
|  |  | # First Aid Kits |  |  |  |
|  |  | # First Responder Kit |  |  |  |
|  |  | # Injuries & Illness Reported |  |  |  |
| Yes | No | # EXCAVATION |  |  |  |
|  |  | # Preconstruction Meeting & Plan |  |  |  |
|  |  | # Slopes & Depths |  |  |  |
|  |  | # Protection |  |  |  |
|  |  | # Ladders & Egress |  |  |  |
| Yes | No | # CRANES |  |  |  |
|  |  | # Engineering |  |  |  |
|  |  | # Crane Certifications |  |  |  |
|  |  | # Secured Access |  |  |  |
|  |  | # Qualified Operators |  |  |  |
|  |  | # Inspections |  |  |  |
| Yes | No | # FALL PROTECTION |  |  |  |
|  |  | # Precon Meeting & Plan |  |  |  |
|  |  | # Leading Edge |  |  |  |
|  |  | # Openings |  |  |  |
|  |  | # Rails & Toe Boards |  |  |  |
|  |  | # Perimeters |  |  |  |
| Yes | No | # SUBCONTRACTORS |  |  |  |
|  |  | # Safety Manuals |  |  |  |
|  |  | # Fall Protection Plan |  |  |  |
|  |  | # Violation History |  |  |  |
| # Safety Manager/Safety Specialist Signature: |  |  |  |  |  |
| # Site Supervisor Signature: |  |  |  |  |  |

COMMENTS:

Copy to:	Project Superintendent; GLY Safety Department

Exhibit 21-2: Example Safety Inspection Open Form

| ##### Project Superintendent: |  |  | ##### Date: |  |
| --- | --- | --- | --- | --- |
| ##### Inspection Conducted by: |  |  | ##### Job Name: |  |
| ##### Submitted to: |  |  | ##### Job Number: |  |

| Note: Ensure all identified safety concerns are addressed appropriately in a timely manner depending on the severity of the concern. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Location | Safety Concern | Recommended Corrective Action | Responsible Contractor | Due Date | Status |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |