22 SAFETY PERFORMANCE MEASUREMENT

22.1 PURPOSE

The purpose of this section is to provide a list of standard safety performance measures that can be used to measure and improve contractor safety performance. The section also provides some guidelines for implementing two leading indicator metrics.

22.2 SAFETY PERFORMANCE INDICATORS

Construction companies are always implementing new risk control strategies to improve their company’s safety performance. However, many do not have an adequate process in place to measure the effectiveness of its risk control strategies. Are these strategies actually helping improve its safety performance? Are there any deficiencies that need to be corrected to prevent accidents and injuries? Safety programs cannot be successful if it does not have a continuous improvement process in place to periodically “check” its effectiveness and recommend solutions to make it even better.

Safety performance indicators will help contractors to check whether their hazard control strategies are working. There are two common types of safety performance indicators used by the construction companies: (a) lagging indicators and (b) leading indicators. Proper selection of lagging and leading indicators to measure their safety performance is critical to the success of the contractor safety program. Effective performance measurement should be capable of measuring compliance and safe/unsafe acts and behaviors.

22.3 LAGGING INDICATORS

Lagging indicators are reactive safety performance measurements that are made after an accident, injury, or illness. They are also called downstream or trailing indicators. Lagging indicators are the most common performance metrics used by construction companies to determine their risks and assess the effectiveness of their company’s safety programs. Even though, lagging indicators does help identify some program deficiencies since the contractor has already suffered a loss, the use of lagging indicators to help improve safety programs has been questioned lately. Example lagging indicators that contractors can consider measuring their past safety performance include:

- OSHA Total Recordable Incident Rate (TRIR)

- OSHA Days Away, Restricted or Transferred Incident Rate (DART)

- Number of days lost per lost time injury

- First Aid Injury Rate (FA)

- Workers’ Compensation Experience Modification Rate (EMR)

- Average cost per indemnity claim

- DOSH citations or inspections with findings

22.4 LEADING INDICATORS

Leading indicators are proactive safety performance measurements that demonstrate the effectiveness of existing safety management systems before an accident, injury, or illness. They are also called upstream indicators. It will help predict future performance by identifying weaknesses or deficiencies in the contractor’s program. Leading indicators are useful tools to assess construction site risks associated with current activities and implement corrective actions immediately before the accident; injury or illness can occur.

Examples of leading indicators, that contractors can consider using to measure safety performance include:

- Accidents Related

  - Percentage of near misses investigated and documented

  - Percentage of incidents reported late

  - Percentage of JHAs changed as a result of accident investigation corrective actions

- Safety Meetings Related

  - Percentage of pre-bid meeting conducted and documented

  - Percentage of pre-construction meeting conducted and documented

  - Percentage of weekly mass safety meeting conducted and documented

  - Percentage of safety committee meeting conducted and documented

- Safety Planning Related

  - Percentage of daily pre-task planning meetings completed and documented

  - Quality of the pre-task plans completed

  - Percentage of daily job hazard analysis completed and documented

  - Quality of the pre-task plans completed

  - Percentage of site specific safety plan completed

  - Quality of site specific safety plan

  - Safety considered during safety look-ahead schedule

  - Additional pre-planning for high-risk evolutions

- Safety Inspections Related

  - Number of the jobsite safety inspections completed with ZERO violations

  - Number of the jobsite audits performed with performance and documentation in compliance

  - Subjective evaluation of the quality of the job site housekeeping

  - Percentage of unsafe behaviors observed

  - Percentage of PPE compliance

  - Number of unsafe conditions observed

  - Percentage of safety violations corrected within a day of identification

- Safety Training Related Compliance

  - Percentage of workers who completed new employee orientation

  - Number of orientations with contractor or owner management participation

  - Percentage of workers with OSHA 10-hr certification

  - Percentage of supervisors with OSHA 30-hr certification

  - Percentage of supervisors with First Aid/CPR certification

- Employee Involvement

  - Worker Safety Perception Survey measuring safety culture

  - Percentage of worker safety suggestions addressed within a week of reporting

  - Number of management project site walks focusing exclusively on safety performance

- Safety Performance Evaluation

  - General contractors evaluated and recognized based on leading indicators (incentive contracts)

  - Subcontractors evaluated and recognized based on leading indicators (incentive contracts)

  - Supervisors evaluated and recognized based on leading indicators

  - Workers evaluated and recognized based on leading indicators

22.5 SELECTION AND IMPLEMENTATION

Incorporating lagging and leading indicators into a contractor safety program should include the following critical steps:

- Educate management at all levels about the importance of safety performance measurement. Obtain their commitment and support to implement a safety performance measurement program at both company and project levels.

- Secure all necessary resources to implement the program: funding, personnel, equipment, and other materials.

- Establish scope and purpose of the program.

- Identify roles and responsibilities – who will champion this program at the company level? Who will be responsible at the project level?

- Hold those responsible for driving the program accountable.

- Select the different types of indicator your company will measure. It is recommended to have a balance of both lagging and leading indicators.

- Develop an implementation plan that addresses the need, timeline, and tools for measurement. Ensure all indicators are measurable, quantifiable, accurate and reliable.

- Train and educate all personnel involved

- Set benchmarks to measure of success for each indicator.

- Develop a distribution or reporting system to share and communicate the progress of the program with the management

- Develop a plan to test the effectiveness of these indicators in measuring company/project safety performance, and whether it meets your company threshold or goal.

- Implement corrective actions if thresholds are not met.

A sample safety performance measure for the project along with goals is presented in Table 22-1. The project team can use similar reporting tool every month or quarter and come up with corrective actions accordingly.

Table 22-1: Example Leading & Lagging Indicators Project Comparison for Company ABC

| Leading Indicators | Goal/ Threshold | Science Building | Goal Met (Y/N) | Corrective Actions? |
| --- | --- | --- | --- | --- |
| Unsafe Conditions: % of safety violations corrected within a day of identification) | 90% | 95% | Y |  |
| Pre-task Plan (PTPs): % of crews completing PTPs | 100% | 75% | N |  |
| Orientation: Percentage of workers who completed new employee orientation | 100% | 90% | N |  |
| Training: Percentage of supervisors with OSHA 30-hr certification | 50% | 60% | Y |  |
| Behavior: % of unsafe behaviors | 0% | 10% | N |  |
| Lagging Indicators |  |  |  |  |
| OSHA Total Recordable Incident Rate (TRIR) | 4.0 | 5.2 | N |  |
| OSHA Days Away, Restricted or Transferred Incident Rate (DART) | 1.0 | 0.5 | Y |  |
| First Aid Injury Rate (FA) | 8.0 | 12.0 | N |  |

22.6 SAMPLE PROGRAM DEVELOPMENT GUIDELINES

Two example checklists presented in the Exhibits 22-1 and 22-2 provides guidance for contractors interested in implementing a Worker Safety Behavior Observation (WSBO), and Pre-Task Plan (PTP) review leading indicator program.

Additional Resources, Readings, and References

- United Kingdom (UK) Health and Safety Executive (HSE)

  - Developing process safety indicators. Available on HSE website for free download http://www.hse.gov.uk/pubns/priced/hsg254.pdf

- The Center for Construction Research and Training (CPWR)

  - Performance Metrics: http://www.cpwr.com/research/performance-metrics

- Construction Industry Institute (CII)

  - Leading Indicators for Safety: https://www.construction-institute.org/scriptcontent/more/284_1_v1-1_more.cfm

  - Project Health Indicator Tool: https://www.construction-institute.org/scriptcontent/more/ir220_2_more.cfm

Exhibit 22-1: Worker Safe Behavior Observation (WSBO) Leading Indicator Program Development Checklist

| Checklist Item | Yes | No |
| --- | --- | --- |
| Contractor has a need to implement a WSBO leading indicator program. |  |  |
| The contractor upper management, including executive's support has been secured. |  |  |
| All necessary resources to implement the WSBO program have been secured: funding, personnel, equipment, and other materials. |  |  |
| The purpose and scope of the program have been established. |  |  |
| The WSBO program will be reviewed during contractor and subcontractor new employee orientation. |  |  |
| A written WSBO program has been developed and were readily available to employee review upon request. |  |  |
| The program outlines the roles and responsibilities of upper management, program owner, observers, and subcontractor management. |  |  |
| The program outlines the observer training requirements that also defines the competence needed for WSBO observers (The observers should, at a minimum possess the knowledge of contractor safety program and all applicable OSHA guidelines and the ability to correct unsafe behaviors). |  |  |
| The trainers are competent to train observers. The trainer competence should be defined in the written program. The training is provided in language trainees will understand. |  |  |
| The program outlines guidelines for the design and development of the WSBO measurement tool – a WSBO card or form. The measurement tool contains the following as a minimum: (1) date and time, (2) observer name and company, (3) location, (4) number of workers' part of the crew being observed, (5) number of workers with safe behavior, (6) number of workers with unsafe behavior, (7) the nature or type of unsafe or safe behavior, and (8) corrective action taken by the observer. Unsafe acts listed on the card/form are tailored to the contractors’ line of work or industry. |  |  |
| The program outlines the observation procedures. Observers should make an observation only for a brief moment (approximately 10 seconds) to verify whether the crew members are performing safe or unsafe behavior (Observers should not stand and stare at the workers, since it might make workers nervous about being watched and portrays observers as safety cops). For example, Figure 1 shows a picture of a worker with improper fall protection. |  |  |
| A minimum of 30 observations is made every week (varies based on the project site). |  |  |
| The WSBO output is quantified by using a “score” for each observation and a “frequency” measurement for the nature of unsafe behavior. The WSBO score is the ratio of workers with safe behavior to the total number of workers observed, multiplied by 100. |  |  |
| The WSBO score is used to assess the severity of the worker behavior on various projects. For trend analysis, the scores are plotted using line graphs weekly or monthly to evaluate safety performance. A sample WSBO score trend analysis is presented in Figure 2 for a hypothetical contractor on three different projects, overall company performance and the contractor’s goal. |  |  |
| The frequencies of the various unsafe behaviors are plotted on a bar graph weekly or monthly to evaluate trends. A sample WSBO unsafe behavior frequency analysis is presented in Figure 3. In this example, the contractor should focus its intervention efforts on hearing protection. |  |  |
| The WSBO program outlines a requirement to set quantified WSBO score objectives. For example, ABC contractor’s goal is to maintain a WSBO score of 90 or above. These objectives should be set, reviewed and modified at appropriate intervals to reflect efforts to achieve continual improvement. |  |  |
| The program outlines the process to introduce safety performance improvement and risk reduction based on “frequency” trends of unsafe behaviors. For example, on ABC contractor’s job sites 35% of workers were not compliant with eye protection. The objective should be set to improve this situation and assign resources to achieve established objectives. |  |  |
| The program outlines the process of communicating WSBO results to the contractor and subcontractor management and all employees. Communication forums include site safety meetings, newsletters, subcontractor coordination meetings, safety committee meetings, and management meetings. |  |  |
| On multi-employer worksites, the contractor has a process in place to communicate individual company WSBO results to appropriate subcontractors. |  |  |
| The program outlines a requirement of corrective action plans for subcontractors with WSBO scores < 50. |  |  |
| A process is in place to incorporate the program requirements in subcontractor contracts to help them budget resources for the WSBO program. |  |  |
| The program outlines data entry and recordkeeping requirements for the measurement tools. |  |  |
| The program outlines incentives to subcontractors or projects with superior WSBO scores. |  |  |
| The program outlines process to review and evaluate the WSBO program every year. |  |  |

Exhibit 22-2: Pre-task Plan (PTP) Quality Leading Indicator Program Development Checklist

| Checklist Item | Yes | No |
| --- | --- | --- |
| Contractor has a need to implement a PTP review leading indicator program. |  |  |
| The contractor upper management, including executive's support has been secured. |  |  |
| All necessary resources to implement the PTP quality review program have been secured: funding, personnel, equipment, and other materials. |  |  |
| The purpose and scope of the program have been established. |  |  |
| The purpose and scope of the PTP quality review program are reviewed during contractor site orientation for all workers. The goal of the PTP review leading indicator program is to prevent injuries/incidents by ensuring that all tasks are planned and implemented per the plan. This helps identify any trends in deficiency and implement suitable intervention methods. |  |  |
| A written PTP quality review program has been developed and is readily available for employee review upon request. |  |  |
| The program outlines the roles and responsibilities of upper management, program owner, reviewers, and subcontractor management. |  |  |
| The program outlines the reviewer training requirements that also define the competence needed for PTP reviewers. The reviewers should, at a minimum possess the knowledge of contractor safety program and all applicable OSHA guidelines. The reviewer should also have the ability to identify hazards and recommend control measures based on work tasks. |  |  |
| The trainers are competent to train reviewers. The trainer competence should be defined in the written program. The training is provided in language trainees will understand. |  |  |
| The program outlines guidelines for the design and development of the PTP review measurement tool – a PTP review card or form. The measurement tool contains the following as a minimum: (1) date and time of review, (2) location, (3) task name or description, (4) PTP adequate or inadequate, and (5) the nature deficiency if the PTP was inadequate. |  |  |
| The program outlines the review procedures. Reviewers should randomly select a PTP from the work site and thoroughly review the plan for adequacy. Reviewers should provide constructive criticism and not blame workers for a poor PTP. This should be considered an educational opportunity. |  |  |
| The PTP review is quantified by using a “score” for each review and a “frequency” measurement for the nature of PTP deficiency. Adequate PTPs should receive a score of 100, and inadequate PTPs will receive zero. For trend analysis, the scores can be plotted using line graphs weekly or monthly to evaluate progress. |  |  |
| The various PTP inadequacies should also be quantified by listing the critical steps for a good PTP in the review card or form: (1) perform the PTP at the work area, (2) identify tasks to be performed in a sequential order, (3) identify the potential hazards associated with each task, (4) identify the control measures for each hazard, (5) review the PTP with crew members, (6) crew members signs the PTP, (7) post PTP in the work area, (8) follow the plan, (9) stop work if there is a change of conditions and make appropriate changes to the PTP, and (10) discuss all changes with the crew members. Figure 4 shows an example of an inadequate pre-task plan. This plan was prepared for lifting equipment off the truck bed and set it inside a building. The plan does not address the essential steps, hazards, and its control measures. The frequency of these deficiencies can be plotted on a bar graph weekly, or monthly to evaluate trends (see Figure 5, for example). |  |  |
| The program outlines the process to retrain crews with inadequate PTPs. |  |  |
| The program outlines a requirement to set quantified PTP score objectives. For example, ABC contractor’s goal is to maintain a PTP score of 95 or above. These objectives should be set, reviewed and modified at appropriate intervals to reflect efforts to achieve continual improvement. |  |  |
| The program outlines the process of communicating PTP review results to the contractor and subcontractor management and all employees. Communication forums include site safety meetings, newsletters, subcontractor coordination meetings, safety committee meetings, and management meetings. |  |  |
| On multi-employer worksites, the contractor has a process in place to communicate individual company PTP results to appropriate subcontractors. |  |  |
| The program outlines a requirement of corrective action plans for subcontractors with PTP scores of less than 75. |  |  |
| A process is in place to incorporate the program requirements in subcontractor contracts to help them budget resources for the PTP program. |  |  |
| The program outlines recordkeeping requirements for the measurement tools. |  |  |
| The program outlines incentives to subcontractor or projects with superior PTP scores. For example, the crew or subcontractor with the best PTP will be rewarded at the safety meeting. |  |  |
| The program outlines the process to review and evaluate the WSBO program every year. |  |  |

Hinze, J. (2005) “A Paradigm Shift: Leading To Safety” 4th Triennial International Conference, Rethinking and Revitalizing Construction Safety, Health, Environment and Quality. Port Elizabeth – South Africa.

Hinze, J. (2005) “A Paradigm Shift: Leading To Safety” 4th Triennial International Conference, Rethinking and Revitalizing Construction Safety, Health, Environment and Quality. Port Elizabeth – South Africa.

Rajendran, S. (2013). “Enhancing Construction Worker Safety Performance Using Leading Indicators.” Periodical on Structural Design and Construction, ASCE, 18 (1), 45-51.

Rajendran, S. (2013). “Enhancing Construction Worker Safety Performance Using Leading Indicators.” Periodical on Structural Design and Construction, ASCE, 18 (1), 45-51.

Rajendran, S. (2013). “Enhancing Construction Worker Safety Performance Using Leading Indicators.” Periodical on Structural Design and Construction, ASCE, 18 (1), 45-51.