23 CONSTRUCTION SITE ENVIRONMENTAL MANAGEMENT

23.1 PURPOSE

Construction site operations have a high potential for impacting the environment. Contractors who fail to control site-related environmental exposures can cause harm to the environment and public health. In order to protect the environment and the public, there are many environmental standards and regulations that impact construction operations both at the federal, state, and local levels. Failure to comply with these requirements can result in enforcement action by regulatory agencies and can also cause employee and public injuries. Furthermore, contractors can be held liable for pollution-related damages resulting in third-party bodily injury and property damage, which can damage their reputation and public image. Therefore, the identification and control of the construction related environmental risks should start as early as the design phase.

This section provides construction contractors some tips to identify and control construction site environmental exposures. Note, simply being unaware of your environmental regulations does not relieve you of your environmental liability.

23.2 PROJECT ENVIRONMENTAL POLICY

Develop a project environmental policy to show a commitment to protect the environment. Most projects integrate it with worker safety and health, to create a “Site Environmental, Safety, and Health Policy.” Ensure this policy is communicated to all site personnel through site orientation and posting in on the safety bulletin board.

23.3 ENVIRONMENTAL COORDINATOR

Designate an Environmental Coordinator (EO) for the project that will help achieve regulatory compliance for the project, and ensure all mitigation measures are properly implemented. The project safety manager assumes this role on medium-scale projects. On large and environmentally complex projects, a full-time environmental manager can be included as part of the project staffing. The project superintendent or project engineer should be the designated environmental representative on small projects. Subcontractors must also provide a designated representative for dealing with environmental issues. At a minimum, the EO should have the knowledge of the site-specific construction activities and associated environmental exposures.

23.4 PRE-CONSTRUCTION SURVEY

Perform an environmental risk assessment and identify all potential environmental exposures and its impacts related to the site’s pre-existing conditions. Consider the following items when evaluating pre-existing site conditions:

  - Identify any environmental contaminants, pollutants, or hazardous waste onsite

  - Look for existing underground tanks or pipelines with the potential to cause environmental damage

  - Evaluate features of adjacent properties that could pose environmental risks to the contractor

  - Test soil and water for contamination

  - Check if the site is environmentally contaminated (brownfield). If so, will the project scope include remediation?

  - Check for the presence of toxic mold onsite if work involves the renovation of existing buildings

See Exhibit 23-1 for an example project environmental checklist from PCL Construction Services, Inc. This is one of the most comprehensive environmental checklists and would be a great tool for smaller contractors to use as a starting point.

  - RISK ASSESSMENT

Perform a detailed environmental risk assessment and identify all potential environmental exposures and its impacts associated with all construction operations including vendors, suppliers, and visitor activities, which will arise at some point during the life of the project. The assessment should consider any adjacent property within or near the work area that could be vulnerable to environmental damage. Some of the common sources of environmental risks associated with construction site operations (including subcontractor activities) include but not limited to:

  - Disturbance to vegetation

  - Rupture of underground tanks or pipelines

  - Discovery of contaminated soil

  - Spills and leaks of contaminants, pollutants, or wastes from onsite operations (e.g., Hydraulic line damage of an excavation equipment)

  - Spills and leaks of contaminants, pollutants, or wastes during transportation (e.g., Release of hazardous waste due to road accident)

  - Air emissions and chemical releases from chemicals and HVAC systems

  - Toxic mold

  - Construction noise

  - Disturbance of lead or asbestos-containing materials

  - Dust (asbestos, silica, etc.)

  - All disposal risks such as improper disposal of both hazardous and non-hazardous waste at job sites contaminated waste (e.g., disposing of pressurized aerosol cans in the garbage dumpster)

23.6 INSURANCE

Examine and identify any coverage gaps in your insurance policies (commercial general liability, business auto, and other related insurance policies) as far as providing protection against the pollution-related liability. All subcontractors and their tiers should be required to carry adequate coverage for risks associated with their operations and require them to add the GC as additional insured.

23.7 DESIGNING FOR THE ENVIRONMENT

Review the environmental risks identified in step 4 and try to eliminate or minimize them through project design by specifying environmentally friendly materials. For example, choose less hazardous and more environmentally friendly products.

23.8 SITE ENVIRONMENTAL MANAGEMENT PLAN (SEMP)

Develop an SEMP integrated into the Site Specific Safety Plan (SSSP) to prevent or minimize any construction effects on the environment. The SEMP should identify all environmental risks, detailed mitigation measures, and procedures to implement them. The SEMP should include at a minimum the following elements:

- Site Environmental Policy

- Roles and Responsibilities

- Permitting Requirements

- Applicable Regulatory and Owner requirements (federal, state, local, and site-specific requirements called in owner contracts)

- Site Specific Risks and Controls

  - Erosion and Sedimentation Controls – to prevent environmental impacts caused by soil erosion and sedimentation.

  - Solid Waste Management Plan – to manage solid wastes associated with onsite construction activities (e.g., concrete, wood, scrap metal, glass, drywall, general trash, etc.). Implement a procedure in place to classify a solid waste as hazardous or non-hazardous.

  - Hazardous Material Controls – to establish procedures for the procurement and management of hazardous materials (e.g., adhesives, paints and thinners, solvents, lead, pesticides, refrigerants coolants such as Freon, etc.). Set up an approval process for any hazardous materials to be brought onsite, and have a policy in place to limit inventory of materials to the time frame (e.g., one week).

  - Hazardous Waste Management Plan – to manage the generation, transport, storage or disposal of any regulated hazardous waste such as used oil, spent solvents, PCBs, asbestos, paint wastes, thinners, etc.

  - Air Pollution Controls – to prevent or minimize air pollution associated with construction activities.

  - Waste Water Management – to manage all construction related wastewater such as truck wheel wash, concrete washout, equipment cleaning, dewatering, cleaning of painting equipment, etc.

  - Spill Prevention and Response Plan – response plan to spills of chemical, fuels, and other substances.

- Environmental Performance Metrics

- Environmental Training

- Inspection Requirements

- Environmental Incident Reporting and Investigation

- Emergency Responses

- Continuous Improvement

23.9 REGULATORY AGENCY RELATIONS

Schedule an onsite meeting with local environmental regulatory agency representatives and go over the project scope and share the SEMP. Apply and obtain “applicable” environmental permits and licenses for the site (e.g., general storm water permit, the Joint Aquatic Resources Permit Application (JARPA), etc.)

23.10 SUBCONTRACTOR SELECTION

Include environmental performance factors as part of the subcontractor safety pre-qualification process. Refer section 3 of this section for more information about subcontractor selection.

23.11 CONTRACTS

Share the SEMP and communicate site environmental requirements for the project with potential bidders through request for proposals (RFP) and pre-bid meetings, so contractors can allocate resources appropriately in the bid. The EO should be involved during the project proposal phase, contract management phase, pre-award and pre-mobilization meetings to ensure that the site environmental expectations are clearly defined and communicated to help project avoid enforcement action. Include site environmental requirements in the contracts. All pre-construction or pre-award meetings should include a review of all environmental standards and regulations, as well as an explanation of project-specific requirements that are above and beyond compliance. Subcontractors should be contractually required to examine their activities and carry out environment risk assessment before commencement of work.

23.12 ENVIRONMENTAL CONTRACTORS

Select a hazardous waste transporter, treatment or disposal contractor. Ensure they have the appropriate permits, licenses, and necessary insurance coverage.

23.13 ENVIRONMENTAL EQUIPMENT

Procure all environmental control related equipment and supplies such as spill response kits, hazardous waste storage shacks/containers, emergency response equipment, and personal protective equipment.

23.14 TRAINING AND EDUCATION

Train workers and supervisors on the general site environmental risk factors and associated mitigation measures, as part of the site’s new employee orientation. Identify and conduct any special training required as part of the project environmental requirements (e.g., hazardous waste operations). Ensure all training records are maintained on site and are readily accessible. The training should also include proper environmental incident reporting procedures and emergency action plan.

23.15 ENVIRONMENTAL PERFORMANCE MONITORING

Develop environmental performance metrics (objectives) similar to worker safety, to improve project environmental performance. The site can measure environmental performance using total points obtained in a weekly site environmental inspection. For example, the inspection results can be a score from 0 to 100 to reflect the environmental compliance of the construction site. Each inspection starts with a score of 100. One point is deducted for a minor non-compliance item (e.g., gap in silt fence), and two points should be deducted for a major violation (e.g., missing catch basin filter). At the end of each inspection, the net scores should be computed and recorded. The project can set a goal to a score of 90 every week.

  - IMPLEMENTATION OF SEMP

Implement the Site Environmental Management Plan (SEMP). Advise all supervisors to consider “environmental risks” as part of their daily pre-task planning.

  - PERFORMANCE MONITORING

Monitor the environmental performance of the project against objectives and the SEMP with the help of regular inspections. Any deficiencies identified during the inspection should be fixed right away. The inspection should focus on the effectiveness and condition of the following environmental controls, but not limited to:

- General project housekeeping

- Construction site entrance cleanliness (e.g., dirt on the road)

- Dust suppression

- Erosion control blankets

- Filter strips

- Mulching

- Silt fencing

- Sediment traps

- Condition of concrete washout areas

- Concrete batch plant

- Equipment fueling area controls (designated area) and containment systems (e.g., berms and dikes)

- Equipment cleaning area

- Equipment maintenance (condition of hydraulic lines and emission control systems)

- Visual inspection of dumpsters to check for waste contamination

- Spill kits

- Hazardous waste containment and cleanup

- Bulk storage area

- Material use and recycling

  - INCIDENT REPORTING AND INVESTIGATION

All personnel should be required to report any and all spills or incidents that occur on site immediately to the site EO or their immediate supervisor. All environmental incidents should be documented and investigated to determine the cause and prevent reoccurrence. See Exhbit 23-2 for an example spill incident form. Ensure the project has the contact numbers of relevant jurisdictional environmental agencies for reporting of environmental incidents. Please refer to the contents of an emergency response in Chapter 17 of this handbook.

23.19 PROGRAM REVIEW

Require the project management to perform a quarterly audit to review the overall effectiveness of the site SEMP, identify and correct any deficiencies.

  - RECORDS RETENTION

Retain all records, reports, permits, self-inspection forms, as part of the project archive.

Additional Resources, Readings, and References

- U.S. Environmental Protection Agency (EPA) – Managing Your Environmental Responsibilities: A Planning Guide for Construction and Development - http://www.epa.gov/compliance/resources/publications/assistance/sectors/constructmyer/myerguide.pdf

- Zurich® - Construction Environmental Best Practices - http://www.zurichna.com/internet/zna/sitecollectiondocuments/en/knowledge%20center/whitepapers/construction/construction_env_best_practices.pdf#page=1

- U.S. Environmental Protection Agency (EPA). Green Buildings. http://www.epa.gov/greenbuilding/index.htm

- Construction Industry Compliance Assistance (CICA). http://www.cicacenter.org/. CICA is your source for plain language explanations of environmental rules for the construction industry. This information is provided free of charge by the National Center for Manufacturing Sciences in partnership with The Associated General Contractors of America (AGC). Funding for this project has been provided by the U.S. Environmental Protection Agency and AGC.

Exhibit 23-1 Example Project Environmental Planning Checklist (page 1)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 2)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 3)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 4)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 5)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 6)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 7)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 8)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 9)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 10)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 11)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 12)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Project Environmental Planning Checklist (page 13)

(Used by permission from PCL Construction Services Inc.)

Exhibit 23-1 Example Environmental Spill Report

(Used by Permission from PCL Construction Services Inc.)