5 PRE-BID AND PRE-CONSTRUCTION MEETINGS

5.1 PURPOSE

This section is intended to provide information and guidelines about conducting the safety portion of the pre-bid and pre-construction meetings. Inclusion and discussion of safety and health in the early stages of the construction project through pre-bid and pre-construction safety meetings will result in better project safety performance.

5.2 PRE-BID MEETINGS

Pre-bid meetings are usually held by the Project Manager or the Contract Manager, in response to a solicitation or request for proposal (RFP). If the company has a safety pre-qualification process in place, the meeting should only be attended by safety pre-qualified bidders. The pre-bid meeting provides an opportunity to the bidders to learn more about the unique challenges or hazards associated with the project and ensure they have a clear understanding of the scope of work requirements including expectations for safety and health performance. The pre-bid meeting will also help clarify any bidder concerns or questions. In order to hold a successful pre-bid meeting the following items should be considered:

- Accept bids only from safety pre-qualified bidders.

- Develop and communicate the meeting agenda, date, time, and location to the bidders at least a week in advance.

- Coordinate a construction site walk-through, which will help the bidders familiarize themselves with the existing project conditions that may have an impact on project safety performance (e.g., Overhead power line runs across the project site).

- Hold the pre-bid meeting at least two weeks in advance of the contract award which will give the bidders enough time to react to any changes or new information provided at the pre-bid meeting.

- Require mandatory pre-bid meeting as a prerequisite for submitting a bid. Document the meeting attendance with the help of a sign-up sheet.

- Assign either the project manager or contract manager as the chairperson to run the meeting.

- Require at a minimum the bidder’s project manager and a safety representative to be in attendance.

- Develop and use a pre-bid meeting safety checklist to review project safety goals and requirements with all prospective bidders. A sample safety requirements checklist template is provided in this section as Exhibit 5-1 that covers the key safety topics that should be discussed during the pre-bid meeting.

- Allow sufficient meeting time to ensure all critical items are covered during the meeting. It is important to know whether there will be a construction site walk-through since it will impact the overall meeting duration.

- Maintain, and publish written pre-bid meeting minutes to all bidders.

5.3 PRE-CONSTRUCTION MEETINGS

Pre-construction meetings provide an opportunity to begin communication about project safety requirements between the contracting parties (e.g., Owner versus General Contractor, General Contractor versus Subcontractors), and address project-specific challenges and hazards. This meeting will provide “detailed” safety direction and guidance to the contractor. The pre-construction meeting is highly recommended for large or complex projects, but will also be beneficial to hold such meetings for smaller projects as well.

The Project Superintendent typically runs the pre-construction meetings, after the contract is awarded, and before the contractor begins work. This meeting is typically included as part of the project mobilization meeting. Consider the following items when planning the pre-construction safety meetings:

- Hold the meeting only after the receipt and review of the contractor’s written safety program. Be prepared to discuss your concerns with the contractor’s safety program and suggest recommended corrective actions.

- Hold the meeting at the project site and be prepared for construction site walk-through.

- Strongly encourage attendance by a majority contractor representatives from both the project management (project manager, cost engineer, and schedule manager) and field management (project superintendent, field superintendent, and General Foreman) including safety.

- Start the meeting by communicating the project’s commitment to worker safety and health.

- Define safety responsibilities

- Require the contractor to submit all the required safety submittals before mobilizing onsite.

- Address all the unique challenges associated with the project and necessary control measures and all the project specific safety requirements.

- Use a pre-construction safety meeting checklist or agenda, to document the safety portion of this meeting as presented in this section as Exhibit 5-2 which covers the key Environmental, Safety, and Health topics to be discussed during the pre-bid meeting. In addition, an example pre-construction/mobilization form from a large construction firm is also presented as Exhibit 5-3.

- Obtain the site safety point of contact and emergency action plan information.

- Receive acknowledgement and commitment to project-specific safety requirements from the contractor by signing the checklist, before commencing work.

Note: The template forms and checklists are long and detailed, but readers are reminded that these are just examples. Contractors can choose to use shorter versions of these forms which can still be very effective.

Exhibit 5-1: SAFETY PRE-BID MEETING CHECKLIST

***Note typically site safety will be one of the several items discussed during the pre-bid meeting***

| Date: ________________________________ Location:_____________________________ |  |
| --- | --- |
| Project Name/Location: _______________________________________________________ |  |
| Contract Package (e.g., Excavation): ____________________________________________ |  |
| Introductions | Yes No NA |
| Route Sign-up Sheet | Yes No NA |
| Project Description & Scope of Work | Yes No NA |
| Staging, Access, Parking, and Use of Facilities | Yes No NA |
| Lunch Tent/Sanitary Facilities | Yes No NA |
| Site Work Hours (start time/end time) | Yes No NA |
| Cleanup and Environmental Considerations | Yes No NA |
| Substance Abuse Program | Yes No NA |
| Required Safety Submittals |  |
| - Safety Personnel Contact | Yes No NA |
| - Written Safety Program | Yes No NA |
| - Corrective Action Plan (CAP) (if required) | Yes No NA |
| - Insurance Coverage Proof | Yes No NA |
| Unique Project Safety Challenges and Hazards |  |
| - Dig Permit | Yes No NA |
| - Air Quality (Odors, Fumes, Noise, Dust, etc.) | Yes No NA |
| - Fire Alarm System and Bypass Requirements | Yes No NA |
| - Lead and Asbestos Abatement/Training | Yes No NA |
| - Public Protection | Yes No NA |
| - PPE requirements | Yes No NA |
| Questions |  |
| Construction Site Walk-through | Yes No NA |

Exhibit 5-2: PRE-CONSTRUCTION MEETING SAFETY CHECKLIST

***Note typically site safety will be one of the several items discussed during the pre-construction meeting***

Date: ________________________________ Location:______________________________

Project Name/Location: _______________________________________________________

Subcontractor Name: _________________________________________________________

The following items were reviewed with the contractor

| Introductions | Yes No NA |
| --- | --- |
| Sign-up Sheet | Yes No NA |
| Project Description & Scope of Work | Yes No NA |
| Project Safety Goals and Commitment | Yes No NA |
| Duties and Responsibilities of Contractors | Yes No NA |
| Staging Area and Job Site Use |  |
| - Trailer or Connex Placement Request | Yes No NA |
| - Site Access – Keys, Passes, hard hat stickers, Badges |  |
| - Parking (Company vs. personal vehicle) | Yes No NA |
| - Deliveries | Yes No NA |
| - Material and/or debris storage and disposal | Yes No NA |
| - Lunch Tent/Sanitary Facilities | Yes No NA |
| - Site Security and Temporary Fencing/Gate Locations | Yes No NA |
| - Typical Site Work Hours Limit (start time/end time) | Yes No NA |
| - After Hour Work Permission Requirements | Yes No NA |
| - Project Supervision Requirements and Notification | Yes No NA |
| - Visitor Restrictions/Sign-in Log | Yes No NA |
| - Dig Permit | Yes No NA |
| - Air Quality (Odors, Fumes, Noise, Dust, etc.) | Yes No NA |
| - Fire Alarm System and Bypass Requirements | Yes No NA |
| - Camera Pass | Yes No NA |
| Medical |  |
| - Drug & Alcohol Testing Requirements | Yes No NA |
| - Occupational Clinic Locations/Map/Phone | Yes No NA |
| - Emergency Plan posted in trailer (muster points, contact lists) | Yes No NA |
| - AED Equipment/First Aid Kit Locations | Yes No NA |
| - Medical Services Requirements | Yes No NA |
| - Commitment to Light Duty /Return to Work Program | Yes No NA |
| Safety & Health Management | Yes No NA |
| - Compliance with DOSH requirements | Yes No NA |
| - Project Safety Staffing | Yes No NA |
| - Accident Reporting, Investigation, and Documentation | Yes No NA |
| - Approved Written Site-Specific Safety Plan | Yes No NA |
| - Project Disciplinary/Inventive Procedures | Yes No NA |
| - Job Hazard Analysis | Yes No NA |
| - Contractor Safety Representative | Yes No NA |
| - New Employee Orientation | Yes No NA |
| - Proper Personal Protective Equipment | Yes No NA |
| - Safety Meetings (type, schedule, & documentation) | Yes No NA |
| - Safety Training & Certification Verification | Yes No NA |
| - Safety Inspections (type, schedule, & documentation) | Yes No NA |
| - Required Weekly Safety Documentation | Yes No NA |
| - Identify DOSH Competent/Qualified Site Supervisor | Yes No NA |
| - Daily Housekeeping/Composite Debris Policy | Yes No NA |
| - Steel Erection Planning | Yes No NA |
| - Crane Inspection/Operator Certification | Yes No NA |
| - English Language /Non-English | Yes No NA |
| - Stretch and Flex | Yes No NA |
| Environmental Hazards | Yes No NA |
| - Runoff Protection | Yes No NA |
| - Hazardous or chemical spill procedures | Yes No NA |
| - Proper cleaning and fueling of equipment onsite | Yes No NA |
| - Concrete Washout Area | Yes No NA |
| - Asbestos, Mold, PCB or lead – Suspected procedures | Yes No NA |
| - Contaminated Soil – Suspected procedures | Yes No NA |
| Unique Project Safety Challenges and Hazards | Yes No NA |
| Questions |  |
| Construction Site Walk-through |  |

Additional Notes/Comments:

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

PRE-CONSTRUCTION SAFETY MEETING ACKNOWLEDGEMENT

We have discussed and understand the Environmental, Safety, and Health requirements of this project.

Contractor agrees to fully comply with all the Environmental, Safety, and Health laws, rules, regulations, and procedures in effect on the site. Contractor agrees to communicate these requirements to any sub-tier contractor(s) brought onsite to complete its scope of work.

| ______________________________________ | ______________________________________ |
| --- | --- |
| Contractor Management (Name) | Contractor Management (Signature) |
| ______________________________________ | ______________________________________ |
| <Company Name> Management (Name) | <Company Name> Management (Signature) |

Exhibit 5-3: Example Contractor Pre-Construction/Mobilization Form

(Use by Permission from SNC-Lavalin)

Note: This best practice tool is only included as an example, and the users should be aware that the contractor may reference safety regulations depending on their jurisdictions. It is the user’s responsibility to ensure that they adapt the best practices provided in this handbook for their jurisdiction and comply with all applicable local, state, and federal regulations affecting their workplace.

We have discussed and understand the Health & Safety requirements of SNC-Lavalin.

_____________________________________

Company Name	Date

_____________________________________

Subcontractor Representative (Name)	Subcontractor Representative (Signature)

| - MOBILIZATION STRATEGY |  |  |  |  |  |  | YES / NO /N/A | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Meetings “We Care”/Safety |  |  |  |  |  |  |  |
| 2 | Daily Tool Box Meetings (all attendees must sign in) |  |  |  |  |  |  |  |
| 3 | Complete JSA’s as appropriate |  |  |  |  |  |  |  |
| 4 | StepBack (team and personal) conducted (all members must sign in and copies retained) |  |  |  |  |  |  |  |
| 5 | Weekly safety meetings to be held |  |  |  |  |  |  |  |
| 6 | Daily work plans to be submitted including type of work planned, manpower, equipment and tools used |  |  |  |  |  |  |  |
| 7 | Develop a site specific risk register |  |  |  |  |  |  |  |
| 8 | Develop a site specific H&S manual |  |  |  |  |  |  |  |
| 9 | Dedicated H&S Coordinator(s) appointed |  |  |  |  |  |  |  |
| 10 | Transportation in / out of site (air / buses) arranged |  |  |  |  |  |  |  |
| 11 | Identification of company employees (e.g. hard hat sticker, badges, etc) |  |  |  |  |  |  |  |
| 12 | Security-access card system |  |  |  |  |  |  |  |
| 13 | First Aid Personnel (number qualified) |  |  |  |  |  |  |  |
|  |  | All supervisors must be qualified |  |  |  |  |  |  |
|  |  | Workers qualified per legislation (e.g. Alberta OHS code part 11) |  |  |  |  |  |  |
| 14 | First Aid Facility and Supplies |  |  |  |  |  |  |  |
|  |  | Any injured employee must be seen by the medical facilities |  |  |  |  |  |  |
|  |  | Immediately reported verbally |  |  |  |  |  |  |
|  |  | Preliminary report completed within 24 hours |  |  |  |  |  |  |
|  |  | Investigation report completed within 7 days |  |  |  |  |  |  |
|  |  | Imminent danger and near misses reported |  |  |  |  |  |  |
| 15 | Vehicles – Proper Safety Equipment and Inspections |  |  |  |  |  |  |  |
|  |  | 20 lb. ABC fire extinguisher |  |  |  |  |  |  |
|  |  | First-aid kit |  |  |  |  |  |  |
|  |  | Back-up alarm (vehicles 1 ton and over) |  |  |  |  |  |  |
|  |  | Company logo visible on both sides of vehicle (minimum 2” height) |  |  |  |  |  |  |
|  |  | Vehicle numbers on the rear window |  |  |  |  |  |  |
| 16 | Vehicles – Registration and Insurance |  |  |  |  |  |  |  |
|  |  | Proof of registration |  |  |  |  |  |  |
|  |  | Minimum $3 million liability insurance for each vehicle |  |  |  |  |  |  |
|  |  | All drivers to have valid a driver’s license |  |  |  |  |  |  |
|  |  | Seat belt mandatory |  |  |  |  |  |  |
|  |  | Gasoline storage and handling |  |  |  |  |  |  |
|  |  | Vehicle pass program |  |  |  |  |  |  |
|  |  | Roll over protection (if applicable) |  |  |  |  |  |  |
| 17 | Site Evacuation |  |  |  |  |  |  |  |
|  |  | Contractor has adequate transportation for full site evacuation of employees |  |  |  |  |  |  |
|  |  | Emergency procedures established and known to all |  |  |  |  |  |  |
|  |  | Muster point |  |  |  |  |  |  |
|  |  | Updated contact lists |  |  |  |  |  |  |
|  |  | Alarm type and procedures |  |  |  |  |  |  |
| 18 | Training |  |  |  |  |  |  |  |
|  |  | Orientation |  |  |  |  |  |  |
|  |  | ID badges, returned for terminated employees |  |  |  |  |  |  |
|  |  | Site training requirements |  |  |  |  |  |  |
|  |  | Employee training log submitted |  |  |  |  |  |  |
|  |  | SNC-Lavalin orientation video shown |  |  |  |  |  |  |
| 19 | Permit Programs in Place |  |  |  |  |  |  |  |
|  |  | LOTO/Isolation |  |  |  |  |  |  |
|  |  | Excavations and earth works |  |  |  |  |  |  |
|  |  | Hot work |  |  |  |  |  |  |
|  |  | Work at height |  |  |  |  |  |  |
|  |  | Confined space |  |  |  |  |  |  |
| 20 | Positive Incentive Program |  |  |  |  |  |  |  |
|  |  | Requirements |  |  |  |  |  |  |
|  |  | Ratios and budget |  |  |  |  |  |  |
|  |  | Participation |  |  |  |  |  |  |
| 21 | Safety Interactions |  |  |  |  |  |  |  |
| 22 | Audits |  |  |  |  |  |  |  |
|  |  | Internal (self-audit) |  |  |  |  |  |  |
|  |  | External (corporate and regulatory audits) |  |  |  |  |  |  |
| - SAFETY EQUIPMENT TO BE MOBILIZED/PROVIDED BY THE CONTRACTOR |  |  |  |  |  |  |  |  |
| 1 | Safety signage |  |  |  |  |  |  |  |
|  |  | Caution |  |  |  |  |  |  |
|  |  | Working above |  |  |  |  |  |  |
|  |  | Caution construction area |  |  |  |  |  |  |
|  |  | Danger open excavation |  |  |  |  |  |  |
|  |  | No smoking |  |  |  |  |  |  |
|  |  | Yellow / red flagging |  |  |  |  |  |  |
|  |  | Confined space |  |  |  |  |  |  |
|  |  | Danger |  |  |  |  |  |  |
|  |  | Power lines |  |  |  |  |  |  |
|  |  | X-ray |  |  |  |  |  |  |
|  |  | Welding in progress |  |  |  |  |  |  |
|  |  | Other |  |  |  |  |  |  |
| 2 | Breathing Apparatus and Fit Testing |  |  |  |  |  |  |  |
|  |  | SABA |  |  |  |  |  |  |
|  |  | SCBA |  |  |  |  |  |  |
| 3 | Fire Fighting Equipment |  |  |  |  |  |  |  |
|  |  | Extinguishers – quantity of 20 lb ABC |  |  |  |  |  |  |
|  |  | Extinguishers – quantity / size of CO2 |  |  |  |  |  |  |
|  |  | Water trucks |  |  |  |  |  |  |
|  |  | Water management / dewatering logs |  |  |  |  |  |  |
| 4 | Safety Harnesses / Shock Absorbing Lanyards |  |  |  |  |  |  |  |
|  |  | Inspection program |  |  |  |  |  |  |
|  |  | Training |  |  |  |  |  |  |
|  |  | Storage |  |  |  |  |  |  |
| 5 | Personal Protective Equipment (as per 6845.3.2.1-EN minimum requirements) |  |  |  |  |  |  |  |
|  |  | Hard hats/helmets, high visibility reflective striping |  |  |  |  |  |  |
|  |  | Safety glasses w/side shields |  |  |  |  |  |  |
|  |  | Safety footwear |  |  |  |  |  |  |
|  |  | High visibility attire (Class I, II, or III) |  |  |  |  |  |  |
|  |  | Hand protection |  |  |  |  |  |  |
|  |  | Hearing protection |  |  |  |  |  |  |
|  |  | Long pants and 4” shirt sleeve |  |  |  |  |  |  |
| 6 | Scaffolding including Tag System and Tags |  |  |  |  |  |  |  |
|  |  | Inspection |  |  |  |  |  |  |
|  |  | Color coding |  |  |  |  |  |  |
|  |  | Certifications |  |  |  |  |  |  |
| 7 | Lockout System in place including Tagging/Sign-out Log Book |  |  |  |  |  |  |  |
|  |  | Procedures |  |  |  |  |  |  |
|  |  | Training |  |  |  |  |  |  |
| 8 | Ladders |  |  |  |  |  |  |  |
|  |  | Procedures |  |  |  |  |  |  |
|  |  | Training |  |  |  |  |  |  |
| 9 | Lunchroom Facilities |  |  |  |  |  |  |  |
| 10 | Toilets (i.e. Alberta OHS Code, Part 24 and Schedule 7) |  |  |  |  |  |  |  |
|  |  | One toilet for 10 persons as a general guide |  |  |  |  |  |  |
|  |  | Additional toilets for women |  |  |  |  |  |  |
| 11 | List of Tools and Equipment, Inspected on a quarterly basis (color code example) |  |  |  |  |  |  |  |
|  |  | Yellow January – March |  |  |  |  |  |  |
|  |  | Green April – June |  |  |  |  |  |  |
|  |  | Red July – September |  |  |  |  |  |  |
|  |  | Blue October – December |  |  |  |  |  |  |
| 12 | Log for disposal of any Hazardous and Non- Hazardous Material |  |  |  |  |  |  |  |
| 13 | Duplicate copy of the MSDS manual |  |  |  |  |  |  |  |
| 14 | Radio, cell phone access |  |  |  |  |  |  |  |
| 15 | Camera passes required on site |  |  |  |  |  |  |  |
| 16 | Other safety equipment (specify) |  |  |  |  |  |  |  |

We have discussed and understand the Health & Safety requirements of SNC-Lavalin.  We will be in full compliance to the SNC-Lavalin Critical Risk Control Protocols at all times unless an Approval for Alternate Controls has been completed and approved where necessary.

Subcontractor agrees to identify minimum training requirements and retain records for the duration of the project for all company workers and contractors.

Subcontractor agrees to provide and ensure the proper use thereof, of all required safety equipment including that specified on this checklist.

________________________________________

Subcontractor Management (Name)	Subcontractor Management (Signature)

_________________________________________

SNC-Lavalin Management (Name)	SNC-Lavalin Management (Signature)

_________________________________________

Project Management (Name)	Project Management (Signature)